TECHNICAL USER MANUAL

# RealHand ROS2 SDK

Installation, Integration, Operation, and API Reference

Table of Contents

1. Project Overview	3

2. Software Architecture	4

3. Repository Structure	4

4. Dependencies	5

5. Installation	5

6. Workspace Setup	6

7. Build Instructions	6

8. Running the SDK	7

9. Launch Files	8

10. ROS 2 Nodes	11

11. ROS 2 Topics	13

12. ROS 2 Parameters	15

13. Message Types	18

14. API Usage	19

15. Common Workflows	21

16. Troubleshooting	22

17. Frequently Asked Questions (FAQ)	22

18. Appendix	23

# 1. Project Overview

The RealHand ROS2 SDK’s nodes construct hand and arm objects from the separately installed realhand Python SDK, adapt their methods to ROS operations, and expose commands and feedback through sensor_msgs/msg/JointState and std_msgs/msg/String.

The following model identifiers are used in this document:

| Device family | Model values used in this document | Command width |
| --- | --- | --- |
| Dexterous hand | L6, O6 | 6 values |
| Dexterous hand | L20 | 16 values |
| Arm | P7, A7lite | 7 values |

## 1.1 Runtime capability boundaries

Repository-verifiable consequences:

Hand metadata confirms command widths and joint order: L6 and O6 use six values, while L20 uses sixteen values.

The touch publisher receives the same force_sensor payload as the force_sensor publisher.

The delivered user-facing capabilities are:

single-hand and dual-hand ROS 2 backends;

typed sensor_msgs/msg/JointState command and state topics;

A7lite and P7 arm ROS 2 backends;

JSON command, sensor, device-information, control-status, and blocking-result topics;

SDK polling, event streaming, cached snapshots, and asynchronous blocking reads;

a PyQt GUI for hand and arm operation;

automated installation helpers and focused example scripts.

# 2. Software Architecture

The package uses a layered design:

## 2.1 Layer responsibilities

| Layer | Implementation | Responsibility |
| --- | --- | --- |
| Model metadata | hand/model.py, arm/model.py | Normalizes model names, defines joint order, and lazily resolves external SDK classes. |
| Adapter | hand/adapter.py, arm/adapter.py | Converts Python SDK objects and sensor records into plain lists/dictionaries and wraps device operations. |
| ROS bridge | hand/node.py, arm/node.py | Declares parameters, creates topics and timers, translates messages, and owns adapter lifetime. |
| GUI ROS facade | gui/ros_client.py | Publishes commands and subscribes to selected feedback topics for the active side. |

## 2.2 Threading and concurrency

Backend ROS callbacks and periodic timers are handled by the rclpy spin executor started with rclpy.spin(node).

The GUI's main ROS client uses a Qt QTimer to call rclpy.spin_once(..., timeout_sec=0.0).

Hand sensor streaming uses a background Python thread to consume the SDK event queue.

Each get_blocking request runs in its own background thread so the ROS callback is not blocked.

GUI auto-detection runs in a separate thread or Qt worker thread.

Stopping a stream requests SDK stream shutdown and waits up to one second for the consumer thread to finish.

# 3. Repository Structure

The current local repository layout is summarized below.

| Path | Current contents / purpose |
| --- | --- |
| examples/ | Hand and arm commands, topic subscriptions, state reading, enable/disable, motion, emergency stop, and backend startup examples. |
| scripts/ | ROS 2 Humble and Jazzy clean-install container launchers and GUI container helpers. |
| launch/ | arm.launch.py, dual_hand.launch.py, gui.launch.py, and hand.launch.py. |
| resource/ | ament package resource marker. |
| src/realhand_ros2/ | hand/, arm/, gui/, py.typed, and package initializer. |
| test/ | Limited model-metadata tests, including joint counts and P7 interface defaults. |
| install.sh | Checks or installs dependencies and the RealHand Python SDK; can optionally install ROS 2, use a local SDK checkout, and create/build a workspace. |
| run_gui.sh | Checks the ROS environment, sources the ROS setup and workspace overlay, and launches the GUI. |
| Repository root | .gitignore, README.md, package.xml, pyproject.toml, setup.cfg, and setup.py. |

# 4. Dependencies

## 4.1 ROS package dependencies

Declared runtime dependencies in package.xml are:

| Dependency | Use |
| --- | --- |
| rclpy | Nodes, publishers, subscribers, parameters, timers, and spinning. |
| launch | ROS 2 launch descriptions and launch arguments. |
| launch_ros | ROS node actions used by launch files. |
| sensor_msgs | sensor_msgs/msg/JointState. |
| std_msgs | std_msgs/msg/String for JSON payloads. |
| python3-pyqt5 | Desktop GUI runtime. |

Test dependencies are ament_copyright, ament_flake8, ament_pep257, and python3-pytest.

## 4.2 Python dependencies

The setup.py install_requires list is setuptools, realhand, and PyQt5. These Python install requirements are distinct from the ROS/runtime dependencies declared in package.xml.

Install a compatible RealHand Python SDK into the same Python environment used by ROS 2.

# 5. Installation

## 5.1 Manual installation

sudo apt update

sudo apt install -y \

git \

iproute2 \

can-utils \

python3-pip \

python3-colcon-common-extensions \

python3-pyqt5

Source ROS 2 and install a compatible RealHand Python SDK into the same Python environment used by ROS 2.

## 5.2 Automated installation

From the current checkout, install dependencies and build a workspace with one of these repository-provided commands:

./install.sh --workspace ~/realhand_ros2_ws
./install.sh --install-ros --workspace ~/realhand_ros2_ws
./install.sh --with-arm --workspace ~/realhand_ros2_ws

Ubuntu 22.04 maps to ROS 2 Humble and Ubuntu 24.04 maps to ROS 2 Jazzy by default. Set ROS_DISTRO explicitly to override the detected default. Use --local-sdk PATH for an editable local RealHand Python SDK checkout.

A7lite kinetix support requires the Python SDK's optional kinetix dependencies, selected by --with-arm. P7 uses the RBot TCP controller and does not require the kinetix extra.

Ubuntu 24.04 uses an externally managed system Python. For direct system-Python installation, the repository adds --break-system-packages and --ignore-installed; do not use pip to upgrade the system pip on Ubuntu 24.04.

# 6. Workspace Setup

Use a conventional ROS 2 workspace layout:

mkdir -p ~/realhand_ros2_ws/src
cd ~/realhand_ros2_ws/src
git clone https://github.com/RealHand-Robotics/realbot-ros2-sdk.git
cd ..

If working from an existing local checkout, copy only source-controlled content. Do not carry old build/, install/, log/, .pytest_cache/, __pycache__/, or *.egg-info/ artifacts into the new workspace.

Every new terminal must source the ROS distribution and, after building, the workspace overlay:

source /opt/ros/humble/setup.bash
source ~/realhand_ros2_ws/install/setup.bash

Optional shell convenience:

echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc
echo 'source ~/realhand_ros2_ws/install/setup.bash' >> ~/.bashrc

# 7. Build Instructions

From the workspace root:

source /opt/ros/humble/setup.bash
cd ~/realhand_ros2_ws
colcon build --symlink-install --packages-select realhand_ros2
source install/setup.bash

Validate the installation:

ros2 pkg prefix realhand_ros2
ros2 pkg executables realhand_ros2

setup.py defines these executable names:

realhand_ros2 realhand_arm_node
realhand_ros2 realhand_gui
realhand_ros2 realhand_hand_node

## 7.1 Run package tests

cd ~/realhand_ros2_ws
colcon test --packages-select realhand_ros2
colcon test-result --verbose

Current automated coverage is limited to part of the model metadata. Within this guide's scope it checks L6 and L20 hand joint counts, A7lite and P7 arm joint counts, and P7's default interface name and interface type. The repository does not include complete hardware communication, topic integration, launch, GUI, or end-to-end tests.

## 7.2 Clean rebuild

When stale generated metadata or an old copied checkout causes discovery problems, remove only the generated workspace directories from the workspace root, then rebuild:

cd ~/realhand_ros2_ws
rm -rf build install log
source /opt/ros/humble/setup.bash
colcon build --symlink-install --packages-select realhand_ros2
source install/setup.bash

# 8. Running the SDK

## 8.1 Prepare SocketCAN

HandNode defaults to can0/socketcan. The A7lite arm model also defaults to can0/socketcan. Backend nodes pass the selected interface values to adapter constructors and do not execute ip link configuration commands.

sudo ip link set can0 down
sudo ip link set can0 type can bitrate 1000000
sudo ip link set can0 up
ip -details link show can0

Useful diagnostics:

ip link show
candump can0

## 8.2 Start a hand backend

source /opt/ros/humble/setup.bash
source ~/realhand_ros2_ws/install/setup.bash
ros2 launch realhand_ros2 hand.launch.py \
  model:=L6 side:=left interface_name:=can0

## 8.3 Start two hands

ros2 launch realhand_ros2 dual_hand.launch.py \
  model:=L6 left_interface:=can0 right_interface:=can1

This launches two realhand_hand_node processes named realhand_left_hand_node and realhand_right_hand_node. Both use the same model argument but different side and interface values.

## 8.4 Start the GUI

ros2 launch realhand_ros2 gui.launch.py

run_gui.sh verifies /opt/ros/$ROS_DISTRO/setup.bash and the repository-local install/setup.bash, sources both, and executes the GUI launch file. It defaults to Humble.

The GUI is initially a ROS client. Use Start Backend on the applicable tab to launch a matching GUI-owned backend, or run the backend separately before opening the GUI. Stop Backend stops the GUI-owned backend process.

## 8.5 Confirm the ROS graph

ros2 node list
ros2 topic list | grep realhand
ros2 node info /realhand_hand_node

# 9. Launch Files

## 9.1 hand.launch.py

Launches one realhand_hand_node named realhand_hand_node with console output.

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| model | L6 | No | Model value passed to HandNode. |
| side | left | No | Topic and SDK side selector: left or right. |
| interface_name | can0 | No | CAN interface passed to the external SDK. |
| interface_type | socketcan | No | External SDK interface type. |
| poll_on_start | true | No | Start supported SDK polling when the Hand adapter is created. |
| stream_on_start | true | No | Start the SDK event stream after node initialization. |
| stream_queue_size | 300 | No | Default maximum queue size for the SDK event stream. |
| poll_intervals_json | {"angle": 0.03333333333333333, "force_sensor": 0.06666666666666667, "torque": 0.2, "speed": 0.5, "acceleration": 0.5, "temperature": 1.0, "current": 0.5, "fault": 1.0} | No | JSON object containing the polling interval in seconds for each supported sensor. |

Note: HandNode directly declares poll_intervals_json with an empty-string default. When started through hand.launch.py or dual_hand.launch.py, Launch passes the preset polling intervals shown above, making them the effective default for Launch-based startup.

## 9.2 dual_hand.launch.py

Launches two hand backends. Both use model and interface_type from common arguments.

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| model | L6 | No | Model applied to both hands. Mixed models are not supported by this launch file. |
| left_interface | can0 | No | Interface for the left hand. |
| right_interface | can1 | No | Interface for the right hand. |
| interface_type | socketcan | No | Interface type for both devices. |
| poll_on_start | true | No | Start supported SDK polling when each Hand adapter is created. |
| stream_on_start | true | No | Start the SDK event stream after each node is initialized. |
| stream_queue_size | 300 | No | Default maximum queue size for each SDK event stream. |

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| poll_intervals_json | {"angle": 0.03333333333333333, "force_sensor": 0.06666666666666667, "torque": 0.2, "speed": 0.5, "acceleration": 0.5, "temperature": 1.0, "current": 0.5, "fault": 1.0} | No | JSON object containing the polling interval in seconds for each supported sensor. |

The polling and streaming values in this table are passed to both the left and right HandNode instances.

The node names are /realhand_left_hand_node and /realhand_right_hand_node. Topic namespaces are generated from each node's side parameter.

## 9.3 arm.launch.py

Launches one realhand_arm_node named realhand_arm_node.

| Argument | Declared default | Required | Meaning / effective default |
| --- | --- | --- | --- |
| model | A7lite | No | Arm model: A7lite or P7. |
| side | left | No | Used by SDK construction and default topic names. |
| interface_name | empty | No | Resolved from model metadata: A7lite -> can0; P7 -> 192.168.10.21. |
| interface_type | empty | No | Resolved from model metadata: A7lite -> socketcan; P7 -> rbot. |
| world_frame | urdf | No | Passed to the external arm constructor. This node does not publish TF. |

## 9.4 gui.launch.py

Launches the realhand_gui executable as node name /realhand_gui.

# 10. ROS 2 Nodes

## 10.1 Hand bridge node

| Item | Description |
| --- | --- |
| Purpose | Exposes one supported RealHand hand through typed command/state and JSON control/sensor topics. |
| Executable | realhand_hand_node |
| Default node name | /realhand_hand_node |
| Input topics | /{prefix}/command (JointState) and /{prefix}/command_json (String). |
| Output topics | Joint state, snapshot, device information, sensor data, blocking results, and control status. |
| Parameters | Model, side, interface, connection, state/poll/stream settings, and command, state, sensor, and status topic overrides. |
| Typical use | Publish position, speed, torque, polling, stream, snapshot, blocking-read, and fault-clear commands and subscribe to feedback. |

Typed Hand JointState fields are processed in this source order: velocity -> HandAdapter.set_speeds(), effort -> HandAdapter.set_torques(), then position -> HandAdapter.set_angles(). JointState.name is informational and is not used to reorder arrays.

Hand JSON processing order is action -> speeds -> torques -> angles. The command field is speeds (plural). An unknown non-empty action raises an unsupported hand JSON action error before array fields are processed.

All HandNode publishers are created for every instance, but optional sensor data depends on the selected model and installed RealHand Python SDK.

### Sensor Polling, Event Streaming, and Blocking Reads

The periodic ROS state timer, SDK polling, SDK event stream, and one-shot blocking reads are separate mechanisms:

| Mechanism | Execution | Output / control |
| --- | --- | --- |
| Periodic ROS state timer | Runs at state_rate_hz; reads the cached snapshot and force-sensor data. | Publishes Hand JointState, snapshot JSON, and related sensor topics. |
| SDK polling | Provided by the external SDK. poll_on_start defaults to true; intervals come from poll_intervals_json or start_polling. | start_polling/stop_polling; supported sensors depend on model and installed SDK. |
| SDK event stream | stream_on_start defaults to true; a background thread consumes an SDK queue whose default maxsize is 300. | Events are routed by type; start_stream/stop_stream control the stream. |
| Blocking read | Each get_blocking request runs once in a separate background thread. | Result is published asynchronously on blocking_result; it is not a ROS service. |

Explicit unsupported polling sensors can return an error. Stopping the stream requests SDK shutdown and waits a finite time for its consumer thread. With pause_polling=true, a blocking read may stop polling temporarily and restore the prior intervals afterward.

## 10.2 Arm bridge node

| Item | Description |
| --- | --- |
| Purpose | Exposes one arm through ROS 2 topics. |
| Executable | realhand_arm_node |
| Default node name | /realhand_arm_node |
| Input topics | /{prefix}/joint_command (JointState) and /{prefix}/command_json (String). |
| Output topics | Joint state, pose, control angle/velocity/acceleration, temperature, moving state, joint limits, and aggregated state snapshot. |
| Parameters | Model, side, model-resolved interface, world frame, TCP offset, connection, state rate, and command/output topic overrides. |
| Typical use | Enable or home an arm, set velocity/acceleration, issue seven-element joint commands, control emergency stop, and monitor typed/JSON state. |

Arm typed JointState fields are processed in this source order: velocity -> ArmAdapter.set_velocities(), effort -> ArmAdapter.set_accelerations(), then position -> move_joints(..., blocking=False). JointState.name does not reorder arrays. When all fields are present, velocity and acceleration are set before the non-blocking position command.

## 10.3 GUI ROS client node

| Item | Description |
| --- | --- |
| Purpose | Provides the ROS communication facade used by the PyQt desktop application. |
| Executable | realhand_gui |
| Node name | /realhand_gui |
| Hand inputs | State, snapshot, device_info, temperature, current, touch, control_status, and blocking_result for the selected side. |
| Hand outputs | Hand command and command_json for the selected side. |
| Arm inputs | Arm joint_state and pose for the selected side. |
| Arm outputs | Arm joint_command and command_json for the selected side. |
| Typical use | Interactive operation, sensor display, presets, auto-grab, slip detection, and starting/stopping a GUI-owned backend process. |

# 11. ROS 2 Topics

All create_publisher() and create_subscription() calls in the backend and GUI nodes pass 10 as the queue‑depth argument.

In the tables below, {side} is left or right. Backend topic names can be replaced with startup parameters when a node is run directly. The GUI client uses hard-coded default topic names and does not discover or follow backend topic overrides.

## 11.1 Hand command topics

| Topic | Type | Direction relative to backend | Description |
| --- | --- | --- | --- |
| /realhand/{side}/hand/command | sensor_msgs/msg/JointState | Subscribe | Typed targets processed velocity, effort, position: speeds, torques, angles. |
| /realhand/{side}/hand/command_json | std_msgs/msg/String | Subscribe | JSON fields speeds, torques, angles; actions clear_faults, start/stop_polling, start/stop_stream, get_snapshot, and get_blocking. |

Typed field mapping:

| JointState field | Hand meaning | Backend handling |
| --- | --- | --- |
| name | Informational only; it does not define command order. | The callback does not read name or reorder arrays. |
| velocity | Speed targets. | HandAdapter.set_speeds(); processed first. |
| effort | Torque targets. | HandAdapter.set_torques(); processed second. |
| position | Angle or position targets. | HandAdapter.set_angles(); processed last. |

The ROS node does not clamp command values or validate vector lengths. Users must supply arrays in the selected model's implementation order; JointState.name does not change that order.

## 11.2 Hand feedback topics

| Topic | Type | Typical source and behavior |
| --- | --- | --- |
| /realhand/{side}/hand/state | sensor_msgs/msg/JointState | Periodic cached angle/speed/torque snapshot. Missing vectors are zero-filled by default. |
| /realhand/{side}/hand/snapshot | std_msgs/msg/String | Periodic JSON conversion of get_snapshot(). |
| /realhand/{side}/hand/angle | std_msgs/msg/String | JSON angle snapshot and/or AngleEvent.data. |
| /realhand/{side}/hand/speed | std_msgs/msg/String | JSON speed snapshot and/or SpeedEvent.data. |
| /realhand/{side}/hand/torque | std_msgs/msg/String | JSON torque snapshot and/or TorqueEvent.data. |
| /realhand/{side}/hand/acceleration | std_msgs/msg/String | JSON acceleration snapshot and/or AccelerationEvent.data, if supported. |
| /realhand/{side}/hand/temperature | std_msgs/msg/String | JSON temperature snapshot and/or TemperatureEvent.data. |
| /realhand/{side}/hand/current | std_msgs/msg/String | JSON current snapshot and/or CurrentEvent.data, if supported. |
| /realhand/{side}/hand/force_sensor | std_msgs/msg/String | JSON force-sensor data. |
| /realhand/{side}/hand/touch | std_msgs/msg/String | Alias publication containing the same force-sensor payload. |
| /realhand/{side}/hand/fault | std_msgs/msg/String | JSON fault snapshot and/or FaultEvent.data, if supported. |
| /realhand/{side}/hand/device_info | std_msgs/msg/String | JSON device metadata; attempted at startup and published again by a one-second timer. |
| /realhand/{side}/hand/blocking_result | std_msgs/msg/String | JSON results for get_snapshot and asynchronous get_blocking requests. |
| /realhand/{side}/hand/control_status | std_msgs/msg/String | Acknowledgements and state for explicitly handled polling and streaming control paths; not a universal JSON error channel. |

The node creates all listed publishers. Optional sensor publishers emit only when the selected model and installed RealHand Python SDK provide data. /hand/touch is an alias publication of the force-sensor payload, not an independent sensor format.

device_info JSON contains available, model, side, interface_name, interface_type, serial_number, firmware_version, mechanical_version, pcb_version, timestamp, and, when unavailable, error. An unavailable device-info read can still publish available=false with empty version fields and an error.

blocking_result payloads follow the source result envelope: action, sensor, request_id, timestamp, ok, and data or error; get_blocking additionally reports timeout_ms and pause_polling. control_status reports acknowledgements and state for explicitly handled polling and streaming control paths. Its payload includes action, ok, timestamp, streaming, polling status, and action-specific fields such as intervals, maxsize, accepted, request_id, sensor, timeout_ms, or error. It is not a universal error channel: most JSON parsing or command-execution exceptions are written only to the ROS log, while get_snapshot and get_blocking results, including errors, are published on blocking_result. An unsupported start_stream operation is an explicit example that publishes control_status with ok=false and an error.

## 11.3 Arm topics

| Topic | Type | Direction relative to backend | Description |
| --- | --- | --- | --- |
| /realhand/{side}/arm/joint_command | sensor_msgs/msg/JointState | Subscribe | Joint positions, velocities, and accelerations. |
| /realhand/{side}/arm/command_json | std_msgs/msg/String | Subscribe | Arm actions and direct command arrays. |
| /realhand/{side}/arm/joint_state | sensor_msgs/msg/JointState | Publish | Seven joint names with positions, velocities, and torque readings in effort. |
| /realhand/{side}/arm/pose | std_msgs/msg/String | Publish | JSON Cartesian pose with x, y, z, rx, ry, and rz. |
| /realhand/{side}/arm/control_angle | std_msgs/msg/String | Publish | JSON SDK control-angle values. |
| /realhand/{side}/arm/control_velocity | std_msgs/msg/String | Publish | JSON SDK control-velocity values. |
| /realhand/{side}/arm/control_acceleration | std_msgs/msg/String | Publish | JSON SDK control-acceleration values. |
| /realhand/{side}/arm/temperature | std_msgs/msg/String | Publish | JSON arm temperature values. |
| /realhand/{side}/arm/moving | std_msgs/msg/String | Publish | JSON object containing is_moving. |
| /realhand/{side}/arm/joint_limits | std_msgs/msg/String | Publish | JSON list with lower and upper for each joint. |
| /realhand/{side}/arm/state_snapshot | std_msgs/msg/String | Publish | Aggregated arm state JSON. |

For arm command JointState, effort carries acceleration commands. For arm feedback JointState, effort carries values returned by the SDK's get_torques(). This asymmetry is important for application design.

Extended topics that call optional SDK methods may produce no messages when the installed RealHand Python SDK or selected arm model does not provide the corresponding method. The bridge does not guarantee all extended states for every model or SDK version.

# 12. ROS 2 Parameters

Parameters are read during node construction. The source does not install parameter-change callbacks, so changing a parameter after startup does not reconfigure the running backend.

## 12.1 Hand node parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| model | string | L6 | Passed to get_hand_model_spec(). |
| side | string | left | Side used by SDK constructor and default topics. |
| interface_name | string | can0 | Device interface. |
| interface_type | string | socketcan | External SDK interface type. |
| connect_on_start | bool | true | If false, topics and timers exist but hardware commands are ignored and no state is published. |
| state_rate_hz | double | 30.0 | Periodic state and snapshot publication timer; zero selects the 30 Hz fallback. |
| fill_missing_fields | bool | true | Zero-fill absent position/velocity/effort vectors to the model joint count. If false, absent vectors are empty. |
| command_topic | string | empty | Empty resolves to /realhand/{side}/hand/command. |
| json_command_topic | string | empty | Empty resolves to /realhand/{side}/hand/command_json. |
| state_topic | string | empty | Empty resolves to /realhand/{side}/hand/state. |
| snapshot_topic | string | empty | Empty resolves to /realhand/{side}/hand/snapshot. |
| angle_topic | string | empty | Empty resolves to /realhand/{side}/hand/angle. |
| speed_topic | string | empty | Empty resolves to /realhand/{side}/hand/speed. |
| torque_topic | string | empty | Empty resolves to /realhand/{side}/hand/torque. |
| acceleration_topic | string | empty | Empty resolves to /realhand/{side}/hand/acceleration. |
| temperature_topic | string | empty | Empty resolves to /realhand/{side}/hand/temperature. |
| current_topic | string | empty | Empty resolves to /realhand/{side}/hand/current. |
| touch_topic | string | empty | Empty resolves to /realhand/{side}/hand/touch. |
| force_sensor_topic | string | empty | Empty resolves to /realhand/{side}/hand/force_sensor. |
| fault_topic | string | empty | Empty resolves to /realhand/{side}/hand/fault. |
| poll_on_start | bool | true | Start supported SDK polling during adapter construction. |
| stream_on_start | bool | true | Start SDK event streaming after node setup. |
| stream_queue_size | integer | 300 | Default SDK event queue maxsize; must be positive when starting a stream. |
| poll_intervals_json | string | empty | Optional JSON object of sensor intervals in seconds; each value must convert to float and be greater than zero. |
| device_info_topic | string | empty | Empty resolves to /realhand/{side}/hand/device_info. |
| blocking_result_topic | string | empty | Empty resolves to /realhand/{side}/hand/blocking_result. |
| control_status_topic | string | empty | Empty resolves to /realhand/{side}/hand/control_status. |

Run directly with parameters not exposed by the launch file:

ros2 run realhand_ros2 realhand_hand_node --ros-args \
  -p model:=L20 \
  -p side:=right \
  -p interface_name:=can1 \
  -p state_rate_hz:=20.0 \
  -p fill_missing_fields:=false

## 12.2 Arm node parameters

| Parameter | Type | Declared default | Description / effective default |
| --- | --- | --- | --- |
| model | string | A7lite | Arm model: A7lite or P7. |
| side | string | left | Used by SDK and default topic names. |
| interface_name | string | empty | Resolved from model metadata: A7lite -> can0; P7 -> 192.168.10.21. |
| interface_type | string | empty | Resolved from model metadata: A7lite -> socketcan; P7 -> rbot. |
| world_frame | string | urdf | Passed to the external arm constructor. |
| tcp_offset | double array | [0.0, 0.0, 0.0] | Passed to the external arm constructor. |
| connect_on_start | bool | true | If false, hardware is not constructed and commands are ignored. |
| state_rate_hz | double | 50.0 | State publication timer; zero falls back to 50 Hz. |
| joint_command_topic | string | empty | Empty resolves to /realhand/{side}/arm/joint_command. |
| json_command_topic | string | empty | Empty resolves to /realhand/{side}/arm/command_json. |
| joint_state_topic | string | empty | Empty resolves to /realhand/{side}/arm/joint_state. |
| pose_topic | string | empty | Empty resolves to /realhand/{side}/arm/pose. |
| control_angle_topic | string | empty | Empty resolves to /realhand/{side}/arm/control_angle. |
| control_velocity_topic | string | empty | Empty resolves to /realhand/{side}/arm/control_velocity. |
| control_acceleration_topic | string | empty | Empty resolves to /realhand/{side}/arm/control_acceleration. |
| temperature_topic | string | empty | Empty resolves to /realhand/{side}/arm/temperature. |
| moving_topic | string | empty | Empty resolves to /realhand/{side}/arm/moving. |
| joint_limits_topic | string | empty | Empty resolves to /realhand/{side}/arm/joint_limits. |
| state_snapshot_topic | string | empty | Empty resolves to /realhand/{side}/arm/state_snapshot. |

## 12.3 Inspect parameters

ros2 param list /realhand_hand_node
ros2 param get /realhand_hand_node model
ros2 param dump /realhand_hand_node

# 13. Message Types

## 13.1 sensor_msgs/msg/JointState

The package uses the standard message without custom extensions.

### Selected hand joint ordering

| Model | Ordered names |
| --- | --- |
| L6, O6 | thumb_flex, thumb_abd, index, middle, ring, pinky |
| L20 | thumb_abd, thumb_yaw, thumb_root1, thumb_tip, index_abd, index_root1, index_tip, middle_abd, middle_root1, middle_tip, ring_abd, ring_root1, ring_tip, pinky_abd, pinky_root1, pinky_tip |

### Arm joint ordering

The supported arm product names in this guide are P7 and A7lite. Both use seven ordered joint names: joint_1 through joint_7.

joint_1, joint_2, joint_3, joint_4, joint_5, joint_6, joint_7

## 13.2 JSON carried in std_msgs/msg/String

The data field contains one JSON value. Command handlers expect a JSON object. Sensor serialization recursively handles dataclasses, Pydantic-like model_dump(), NumPy-like tolist(), SDK to_list(), dictionaries, lists, tuples, primitives, and finally repr().

### Hand JSON commands

| Payload field/action | Effect and validation |
| --- | --- |
| speeds | Calls set_speeds; processed after action. |
| torques | Calls set_torques; processed after speeds. |
| angles | Calls set_angles; processed after torques. |
| action: clear_faults | Calls the adapter fault-clear operation. Availability depends on the selected model and installed RealHand Python SDK. |
| action: start_polling | Optional intervals must be a JSON object; every value must convert to float and be > 0. Explicit unsupported sensors may return an error. |
| action: stop_polling | Stops SDK polling and publishes status on control_status. |
| action: start_stream | Optional maxsize is converted to an integer and must be positive; starts background SDK-event consumption. |
| action: stop_stream | Requests SDK stream stop, ends the consumer thread, and publishes status. |
| action: get_snapshot | Optional sensor and request_id; publishes the result on blocking_result. |
| action: get_blocking | Requires sensor; accepts timeout_ms, pause_polling, and request_id; runs in a worker thread and publishes asynchronously on blocking_result. |

A single Hand JSON object is handled in the order action -> speeds -> torques -> angles. Unknown non-empty actions are errors, so their array fields are not reached. Supported sensors and fault/poll/stream operations depend on the selected model and installed RealHand Python SDK.

### Arm JSON commands

| Payload field/action | Effect |
| --- | --- |
| velocities | Calls set_velocities; processed first. |
| accelerations | Calls set_accelerations; processed second. |
| action: enable | Calls enable(). |
| action: disable | Calls disable(). |
| action: home | Calls home(blocking=...). |
| action: reset_error | Calls reset_error(). |
| action: emergency_stop | Calls emergency_stop(enable); enable defaults to true. |
| action: resume_from_emergency_stop | Uses SDK resume_from_emergency_stop() when available, otherwise emergency_stop(False); raises an unsupported-resume error if neither exists. |
| joints | Calls move_j(values, blocking=...); processed after action. |

Arm JSON order is velocities -> accelerations -> action -> joints. Thus motion settings are applied before an action, and joints are handled afterward. home and joints accept blocking; emergency_stop accepts enable. Use true JSON booleans: do not send the string "false", because Python bool("false") is true. An unknown action itself is ignored, while recognized velocities, accelerations, and joints in the same object still execute.

# 14. API Usage

## 14.1 Publish a typed hand command

python3 examples/send_hand_command.py --model L6 --side left --positions 50,50,50,50,50,50
python3 examples/send_hand_command.py --model L6 --side left --velocities 40,40,40,40,40,40
python3 examples/send_hand_command.py --model L6 --side left --torques 30,30,30,30,30,30

If a typed command contains all three arrays, the backend applies velocity, then effort, then position. Arrays must follow the selected model's implementation order; name is informational, and the ROS node adds no clamping or vector-length validation.

## 14.2 Hand JSON control and reads

The example publisher accepts raw JSON and exposes every Hand action:

python3 examples/send_hand_command.py --side left --json '{"action": "clear_faults"}'
python3 examples/send_hand_command.py --side left --json '{"action": "start_polling", "intervals": {"angle": 0.03333333333333333}}'
python3 examples/send_hand_command.py --side left --json '{"action": "stop_polling"}'
python3 examples/send_hand_command.py --side left --json '{"action": "start_stream", "maxsize": 300}'
python3 examples/send_hand_command.py --side left --json '{"action": "stop_stream"}'
python3 examples/send_hand_command.py --side left --json '{"action": "get_snapshot", "sensor": "angle", "request_id": "angle-cache-1"}'
python3 examples/send_hand_command.py --side left --json '{"action": "get_blocking", "sensor": "fault", "timeout_ms": 500, "pause_polling": true, "request_id": "fault-1"}'

get_snapshot and get_blocking publish results on /realhand/left/hand/blocking_result. get_blocking is asynchronous topic-based processing, not a synchronous ROS service.

## 14.3 Arm startup and movement

# Start model-correct backends.
./examples/start_backend.sh arm A7lite left can0
./examples/start_backend.sh arm P7 left 192.168.10.21 rbot

# Enable the arm.
python3 examples/arm_enable_disable.py --side left enable

# Set velocity and acceleration.
python3 examples/arm_set_velocity_acceleration.py --model P7 --side left \
  --velocities 0.5,0.5,0.5,0.5,0.5,0.5,0.5 \
  --accelerations 1,1,1,1,1,1,1

# Execute move_j; motion settings in this JSON object run first.
python3 examples/arm_move_j.py --model P7 --side left \
  --joints 0,0,0,0,0,0,0 \
  --velocities 0.5,0.5,0.5,0.5,0.5,0.5,0.5 \
  --accelerations 1,1,1,1,1,1,1

Arm command processing is velocity -> acceleration -> position for typed JointState, and velocities -> accelerations -> action -> joints for JSON. Position movement uses non-blocking move_j for typed commands.

## 14.4 Arm state and emergency-stop examples

python3 examples/arm_read_state.py --side left
python3 examples/arm_emergency_stop.py --side left
python3 examples/arm_enable_disable.py --side left resume_from_emergency_stop
bash examples/arm_move_then_emergency_stop.sh left 0.5

arm_read_state.py reads joint_state and pose. The final shell example sends a move command and triggers emergency_stop after the requested delay.

## 14.5 Direct adapter API

The adapters are importable Python classes:

from realhand_ros2.hand import HandAdapter

hand = HandAdapter(
    model="L6",
    side="left",
    interface_name="can0",
    interface_type="socketcan",
)
try:
    hand.set_speeds([40] * 6)
    hand.set_torques([30] * 6)
    hand.set_angles([50] * 6)
    print(hand.snapshot_payload())
finally:
    hand.close()

## 14.6 Example script entry points

| Entry point | Purpose |
| --- | --- |
| examples/start_backend.sh | Start one hand, one arm, or a dual-hand backend. |
| examples/read_topics.sh | List, inspect, or echo Hand and Arm topics. |
| examples/send_hand_command.py | Send typed Hand position/speed/torque commands or raw Hand JSON. |
| examples/subscribe_hand_topics.py | Subscribe to core or all Hand feedback topics. |
| examples/send_arm_command.py | Send typed Arm arrays or Arm JSON actions. |
| examples/subscribe_arm_topics.py | Subscribe to Arm joint_state and pose topics only; this example does not subscribe to the extended Arm output topics. |
| examples/arm_enable_disable.py | Enable, disable, reset errors, or resume from emergency stop. |
| examples/arm_set_velocity_acceleration.py | Set seven velocity and acceleration values. |
| examples/arm_move_j.py | Send move_j with optional velocity, acceleration, and blocking. |
| examples/arm_read_state.py | Read one joint_state and pose sample. |
| examples/arm_emergency_stop.py | Send emergency_stop. |
| examples/arm_move_then_emergency_stop.py | Publish a validated Arm JointState move command, wait for the configured delay, and then publish emergency_stop. |
| examples/arm_move_then_emergency_stop.sh | Move, wait for a configured delay, then emergency stop. |

These examples validate their own command inputs where implemented, but backend capabilities still depend on the selected device and installed RealHand Python SDK.

# 15. Common Workflows

## 15.1 First hand bring-up

1. Configure can0 for the selected hand.

2. Source ROS 2 and the workspace overlay.

3. Start L6: ./examples/start_backend.sh hand L6 left can0

4. Read topics: ./examples/read_topics.sh hand left echo-once

## 15.2 Dual-hand operation

Launch dual_hand.launch.py, then inspect the two topic namespaces:

The provided dual launch supports a common model only. For different left/right models, start two hand.launch.py processes with distinct model, side, and interface arguments.

## 15.3 GUI hand workflow

The GUI checks for changed hand positions every 100 ms and publishes only when values differ; preset and home actions publish immediately. Auto Grab uses per-finger baseline subtraction and two-sample threshold debounce.

## 15.4 Slip detection and torque boost

The GUI computes touch magnitude change and pressure-centroid displacement. When both configured thresholds are crossed outside the cooldown window, it marks a slip. Optional torque boost increments command values for joints mapped to that finger and clamps them to 100.

## 15.5 Polling, streaming, snapshots, and blocking reads

Use start_polling/stop_polling for SDK polling, start_stream/stop_stream for the SDK event queue, and watch control_status for acknowledgements and state. Use get_snapshot for a cached read and get_blocking for a one-shot asynchronous blocking read whose result arrives on blocking_result. Sensor availability depends on the model and installed RealHand Python SDK.

ros2 topic echo /realhand/left/hand/control_status
ros2 topic echo /realhand/left/hand/blocking_result
python3 examples/send_hand_command.py --side left --json '{"action": "start_polling"}'
python3 examples/send_hand_command.py --side left --json '{"action": "start_stream", "maxsize": 300}'
python3 examples/send_hand_command.py --side left --json '{"action": "get_snapshot", "sensor": "angle", "request_id": "snapshot-1"}'
python3 examples/send_hand_command.py --side left --json '{"action": "get_blocking", "sensor": "force_sensor", "timeout_ms": 1000, "pause_polling": true, "request_id": "force-1"}'
python3 examples/send_hand_command.py --side left --json '{"action": "stop_stream"}'
python3 examples/send_hand_command.py --side left --json '{"action": "stop_polling"}'

## 15.6 Arm motion and emergency stop

Start A7lite with can0/socketcan or P7 with its TCP/IP rbot interface. Enable the arm before motion, apply velocity and acceleration, send move_j, and use the focused emergency-stop and resume examples:

./examples/start_backend.sh arm A7lite left can0
./examples/start_backend.sh arm P7 left 192.168.10.21 rbot
python3 examples/arm_enable_disable.py --side left enable
python3 examples/arm_set_velocity_acceleration.py --model P7 --side left --velocities 0.5,0.5,0.5,0.5,0.5,0.5,0.5 --accelerations 1,1,1,1,1,1,1
python3 examples/arm_move_j.py --model P7 --side left --joints 0,0,0,0,0,0,0
python3 examples/arm_read_state.py --side left
python3 examples/arm_emergency_stop.py --side left
python3 examples/arm_enable_disable.py --side left resume_from_emergency_stop
bash examples/arm_move_then_emergency_stop.sh left 0.5

# 16. Troubleshooting

## 16.1 Command fails or moves unexpected joints

Confirm model and array length.

Use the exact implementation-defined order in Section 13.

Do not assume JointState.name causes reordering; it does not.

Remember that arm command effort means acceleration.

# 17. Frequently Asked Questions (FAQ)

## Does JointState.name determine command ordering?

No. Both backend callbacks pass numeric arrays directly. Use the exact order for the selected model.

## What does JointState.effort mean?

For hand commands it is torque. For hand state it is the cached torque vector. For arm commands it is acceleration. For arm state it is the SDK torque reading.

## Can I use different models in dual_hand.launch.py?

No. That launch file has one common model argument. Launch two single-hand instances for mixed models.

## Can I change parameters at runtime?

The nodes read operational parameter values during construction and do not register a parameter-change callback.

## How do I select right-side topics?

Launch with side:=right and use /realhand/right/.... The GUI recreates its topics when the side selection changes.

## Can I disable hardware connection for graph testing?

Yes, when running the node directly, set connect_on_start:=false. Topics exist and commands are ignored with warnings, but no simulated state is published.

# 18. Appendix

## 18.1 Package metadata

| Item | Value |
| --- | --- |
| Package name | realhand_ros2 |
| Version | 0.1.0 |
| Maintainer | realhand |
| Maintainer email | realhand@todo.todo (placeholder value in source) |
| Typed package marker | src/realhand_ros2/py.typed |

## 18.2 Console entry points

| Executable | Python entry point |
| --- | --- |
| realhand_hand_node | realhand_ros2.hand.node:main |
| realhand_arm_node | realhand_ros2.arm.node:main |
| realhand_gui | realhand_ros2.gui.app:main |

## 18.3 GUI operational defaults

| Setting | Default |
| --- | --- |
| Hand position publish check | 100 ms |
| Live speed/torque debounce | 100 ms |
| Preset cycle period | 1,000 ms |

## 18.4 Useful inspection commands

ros2 pkg prefix realhand_ros2
ros2 pkg executables realhand_ros2
ros2 node list
ros2 topic list -t
ros2 topic info -v /realhand/left/hand/state
ros2 topic hz /realhand/left/hand/state
ros2 param dump /realhand_hand_node
ros2 service list
ros2 action list
ip -details -statistics link show can0

End of document